# North Lincolnshire Council

Daily Event Analysis

1st November 2010

# Overview

The Low Santon TEOM measured a daily mean PM10 concentration of 51µg/m3 on Monday 1st November 2010 exceeding the allowed 50µg/m3 as a daily mean. The Dawes Lane Coke Oven (DLCO) Osiris measured a daily PM10 concentration of 45g/m3. A check of the data has revealed that the PM10 concentrations were significantly above the 50g/m3 threshold between 14:30 and 16:45 at the Low Santon TEOM, the same period at the DLCO Osiris measured concentrations marginally above the daily objective limit. The average PM10 concentration for this time period at the Low Santon TEOM was 147g/m3. The DLCO Osiris averaged at 45g/m3.

# Met Conditions

Winds ranged between 180° & 10°. The wind within the 2 hour & 15 min period of focus ranged between 202° and 228°. Wind speeds averaged at 2.8 m/s. Heavy rain was observed in the evening after 19:00.

# Episode Wind Range

# Potential Sources

The above map shows the Low Santon TEOM (indicated by the red lines) and the DLCO Osiris (indicated by the green). The black line within the red shows the wind direction of the highest concentration recorded during the 2 hour  and 15 min period of focus.

As the Low Santon TEOM measured consistently higher concentrations during this period, it is likely that the TEOM was affected by a different source to the DLCO Osiris. Therefore, the areas in which the red and green lines intersect are unlikely to be the location of the problem.

The likely areas include;

- Redbourn Site

- Slag Haul Road

- Intermediate Site

The black line indicates the highest measured concentration of 212g/m3 recorded at the Low Santon TEOM during this period. It runs along the Slag Haul Road, Redbourn Site and the Intermediate Site.